# Temperature and Pressure Monitoring and Alarm System Design

High Level Design:

- Initial stage system will read the configuration required.

- Create threads for read and process the temperature reading

- Monitor thread will read the temperature or pressure based on the configuration and update the common data structure

- The Processing thread will parse the threshold configuration for both temperature and pressure.

- Compare the read data with threshold and give warning, if the temperature exceeding will continue for sampling time set by the configuration.

Low Level Design Flow:

Polling Thread Config structure:

- Parameter

- Read function

- Polling time

- Last polling time

Process Thread Config structure:

- Parameter

- lower threshold

- higher threshold

- Check time interval

- Last check time interval

Common database structure

- Parameter

- Read value

Temperature Low threshold :-  -20 °C

Temperature Higher Threshold : 70 °C

Pressure Low Threshold : 0 PSI

Pressure High Threshold : 100 PSI


Response interval > polling time

Structure declarations:

Typedef enum

{

Param_Temp, 	// temperature parameter ID

Param_Pressure, 	// Pressure parameter ID
	Param_Config 	// Configuration ID

} polling_param;

typedef struct

{

polling_param polling_param_t; // Input – for distinguish which parameter

Int iReadValue; 	// store the read value corresponds to each parameter

}read_data;

typedef struct

{

Polling_param polling_param_t; // param ID

int (*readfn)(void); // function pointer, will invoke corresponding function

long int polling_time_in_ms; // constant read from config structure

long int last_poll_time_ms; // update after each polling

}polling_thread_config;

typedef struct

{

Polling_param polling_param_t; // param ID

int iHigherThreshold;  // constant value read from configuration

int iLowerThreashold; // constant value read from configuration

long int liCheckingTime; // constant value read from configuration

long int liLastCheckTime; // to store the last check time

}processing_thread_config;

Code flow:

- Declare two thread handles

- Create two thread for monitoring and processing

Monitoring thread:

- Read the monitoring thread configuration structure

- If the difference between the previous read time and present read time greater than the polling time, the thread will call the corresponding read function.

- Read function

  - Read the random value

  - Return the read value

- Assign the parameter value and read value to the common read structure

Processing thread:

- Read the configuration structure for processing thread.

- If the difference between the current check time and last check time greater than config check time move to next step, else no process

- Parse the common structure data

- Check the read value parsed from the structure is exceeds the threshold range.

- If value is exceeding the threshold, print the warning message else no process.

Declare two thread handle

Create monitor and process threads using Pthread_create

Get the current time in ms using gettimeofday function and store it to current_polling_time

Load the polling time value, parameter ID and function

Start parsing monitor thread config structure using loop

Start parsing monitor thread config structure using loop

Parse Parameter ID, checking interval, last checking interval, upper and lower threshold values

Check difference between current_poll_time and last_poll_time greater than poll_time

Get the current time using gettimeofday function

No

No

Set the current time to last_poll_time

Yes

No

Store the parameter ID and and read value to common structure

Read parameter values and return from read function

Call the read function

Check the difference between current time and last check time with check interval

Yes

Parse the common data read structure

Set the current time to last check time

Check the Read parameter value exceeds the range from lower threshold to upper threshold

Yes

Print the warning message

dghj